# Matrice 4T + D-RTK 3 — Data & Architecture Reference

Prepared for Drone Catalyst · Survey Operations / FlightHub integration planning · June 2026

This document inventories the data types available from a DJI Matrice 4T aircraft with a D-RTK 3 multifunctional base station, as delivered through DJI Pilot 2, FlightHub 2, and the DJI Cloud API. It contrasts this real-world stack with the current Drone Catalyst /testflighthub sandbox API.

## 1. How data reaches your platform

| Channel | Protocol | Rate | Typical use |
| --- | --- | --- | --- |
| OSD telemetry | MQTT thing/product/{device_sn}/osd | ~0.5 Hz (every 2s) | Live position, battery, cameras, RTK state |
| State events | MQTT thing/product/{device_sn}/state | Event-driven | Mode changes, errors, config updates |
| FlightHub 2 REST API | HTTPS OpenAPI | On demand | Device list, commands, media, HMS, RTK calibration |
| Live video | RTMP / WebRTC / GB28181 | Continuous | Pilot view (not raw sensor pixels) |
| Media files | Cloud upload / SD export | Per capture | Photos, R-JPEG thermal, MP4, flight logs |
| D-RTK 3 corrections | Radio RTCM to aircraft | Continuous | Centimeter positioning |
| D-RTK 3 raw logs | USB export .DAT | Logged locally | PPK in DJI Terra / RINEX conversion |

Setup path: DJI Pilot 2 → Cloud Services → your platform URL + MQTT broker.

## 2. Position, navigation & flight path (aircraft OSD)

These fields build a flight path when stored as a time series. Flight path = ordered (latitude, longitude, timestamp) points.

| Field | Description |
| --- | --- |
| latitude / longitude | WGS84 coordinates |
| height | Height above takeoff point (m) |
| elevation | Ellipsoid / AMSL-related elevation |
| horizontal_speed / vertical_speed | Ground and vertical velocity |
| attitude_head / pitch / roll | Aircraft orientation (°) |
| home_latitude / home_longitude / home_distance | Home point and distance to home |
| track_id | Current flight/track identifier |
| total_flight_distance / time / sorties | Lifetime flight statistics |

## 3. RTK / GNSS state (D-RTK 3 effect on aircraft)

The D-RTK 3 base broadcasts RTCM corrections. The Matrice 4T reports fix quality via position_state in OSD.

| Field | Description |
| --- | --- |
| position_state.gps_number | GPS satellites tracked |
| position_state.rtk_number | RTK satellites used |
| position_state.is_fixed | not_started / fixing / fixing_successful / fixing_failed |
| position_state.quality | gear_1 … gear_5 or rtk_fixed |
| mode_code | Includes airborne_rtk_fixing_mode |

### D-RTK 3 base station data products

| Mode | Data product | Typical accuracy |
| --- | --- | --- |
| Base station (broadcast) | RTCM 3.x corrections to multiple aircraft | Relative cm-level to base |
| Uncalibrated base | Single-point positioning | ~1.5 m H / 3.0 m V RMS |
| Satellite differential | After ~20 min convergence | ~30 cm H / 40 cm V RMS |
| Network RTK calibrated | NTRIP or surveyed coordinates | ~1 cm H / 3 cm V + 1 ppm |
| Raw observation log | RTCM 3.2 .DAT files | PPK in DJI Terra |
| Rover mode | Survey points | JSON mark file / CSV checkpoints |
| Relay mode | O4 Enterprise radio relay | Link reliability (not positioning) |

## 4. Flight status & safety

- mode_code — standby, manual flight, wayline, RTH, landing, tracking, POI, etc.

- mode_code_reason — low battery, RC lost, obstacle on RTH, hardware fault, etc.

- gear, locked, rc_lost_action, rth_altitude, rth_mode, current_rth_mode

- commander_flight_mode / height / lost_action

- distance_limit_status — max distance, near-limit flag, enabled state

- height_limit, is_near_height_limit, is_near_area_limit, geo_caging_status

- obstacle_avoidance — horizon / upside / downside sensing on/off

- night_lights_state, wind_speed, wind_direction

- low_battery_warning_threshold, serious_low_battery_warning_threshold

## 5. Battery & power

| Field | Description |
| --- | --- |
| battery.capacity_percent | Total remaining % |
| battery.remain_flight_time | Estimated seconds left |
| battery.return_home_power | RTH trigger % |
| battery.landing_power | Forced landing % |
| battery.batteries[] | Per-battery: index, sn, capacity_percent, voltage (mV), temperature (°C), loop_times, firmware_version, high_voltage_storage_days |

## 6. Device identity, storage & maintenance

- activation_time, firmware_version, firmware_upgrade_status, compatible_status

- control_source — RC / app / cloud control owner

- storage.total / storage.used — onboard storage (KB)

- maintain_status.maintain_status_array[] — maintenance history and due state

- offline_map_enable

## 7. Payload & gimbal

| Field | Description |
| --- | --- |
| payloads[] | payload_index, sn, firmware_version, control_source |
| type_subtype_gimbalindex | gimbal_pitch, gimbal_roll, gimbal_yaw, zoom_factor |

## 8. Matrice 4T camera / sensor telemetry (cameras[])

The M4T integrates wide + medium tele + telephoto + thermal + laser rangefinder. OSD provides status and settings, not full pixel streams.

### Common per-camera fields

- camera_mode, photo_state, recording_state, record_time

- remain_photo_num, remain_record_duration, liveview_world_region, screen_split_enable

### Wide-angle camera

- wide_exposure_mode, wide_exposure_value, wide_iso, wide_shutter_speed

### Zoom / telephoto camera

- zoom_factor, zoom_exposure_mode, zoom_exposure_value, zoom_iso, zoom_shutter_speed

- zoom_focus_mode, zoom_focus_state, zoom_focus_value, zoom_calibrate_*

### Thermal (M4T-specific)

| Field | Description |
| --- | --- |
| ir_zoom_factor | Thermal zoom ratio |
| ir_metering_mode | Off / spot / area temperature measurement |
| ir_metering_point | x, y (0–1), temperature (°C) |
| ir_metering_area | ROI bounds, aver_temperature, min/max temperature points |

Hardware thermal specs: 640×512 VOx, high gain −20°C to 150°C, low gain 0°C to 550°C, ±2°C or ±2% accuracy, R-JPEG radiometric photos.

### Laser rangefinder (M4T hardware)

Hardware: range up to 1800 m, accuracy ±(0.2 + 0.0015×D) m. Not a dedicated continuous OSD field in the Matrice 4 Series schema; target distance/GPS coordinates appear in capture metadata and Pilot UI.

## 9. Connectivity & accessories

- dongle_infos[] — IMEI, EID, SIM/eSIM state, carrier, ICCID

- camera_watermark_settings — datetime, SN, GPS, custom text overlay toggles and layout

## 10. Other data types (full stack)

| Type | Examples |
| --- | --- |
| HMS health messages | Motor, compass, IMU, RTK, battery warnings |
| Live video metadata | Stream ID, quality, error status (live_status[]) |
| Media files | Wide/tele JPG, R-JPEG thermal, MP4, SRT sidecars |
| Flight logs | DJI flight records from aircraft storage |
| Wayline / mission | KMZ/WPMZ missions, progress events |
| PPK / RTK post-process | D-RTK .DAT → Terra → refined image positions |
| Survey outputs (D-RTK rover) | GCP mark JSON, checkpoint CSV |

## 11. What Matrice 4T does NOT provide

- LiDAR point clouds — no Zenmuse L2/L3; fixed payload only

- Interchangeable H30T payload — H30T is for M350/M400, not M4T

- Full thermal frame over MQTT — only spot/area temps + files on capture

- Raw RTCM stream in FlightHub OSD — corrections are radio-side; aircraft reports fix state

- Continuous laser range in standard OSD schema

## 12. Comparison: Drone Catalyst sandbox vs real stack

Current /testflighthub API persists 10 fields per telemetry tick:

- drone_id, timestamp, latitude, longitude, altitude, speed, battery, status

- Plus DB metadata: id, created_at

A real Matrice 4T + D-RTK 3 + FlightHub 2 integration exposes 100+ distinct telemetry properties across OSD/state, plus separate media and log channels — including RTK quality, dual batteries, thermal spot temps, gimbal attitude, wind, obstacle sensing, and mode/reason codes.

## 13. References

DJI Matrice 4 Series specs

DJI D-RTK 3 specs

FlightHub 2 — Matrice 4 Series OpenAPI schema

DJI Cloud API — device properties & telemetry

Drone Catalyst test environment